AD600NA6L（R100）

Product Feature:

- Onboard Intel 12th/13th-U/-P serial processor

- 2*DDR5 SO-DIMM，max support 64GB

- 6*2.5G lan port（5*Intel i226-V，1*RTL8156B）

- TPM2.0，2*M.2 2280 SSD，M.2 WIFI

- 1*HDMI2.0，1*DP，Type-C（USB3.0+DP）

- DC 12-19V；Size：120x120mm

Product Specification

| Processor System | - Onboard Intel 12th/13th-U/-P series processor, TDP 35W |
| --- | --- |
|  | - EFI BIOS |
| Memory | - 2*DDR5 SO-DIMM, total max 64GB |
| Storage | - 2*M.2 M-Key 2280(NVMe PCIe 4.0 x4/SATA3.0 protocol), order notes |
|  | - 1*SATA3.0, 2Pin 5V power supply |
| Display | - 1*HDMI2.0, support 4096x2160@60Hz |
|  | - 1*DP, support 7680x4320@60Hz |
|  | - 1*Type-C port, support DP 7680x4320@60Hz和USB3.2 Gen2 |
| Board Side I/O Interface | - Power on button, reset button, 3*USB3.0, 1*USB2.0 |
|  | - DC JACK, 1*HDMI, 1*DP, 1*Type-C (USB3.0+DP) |
|  | - 6*LAN, 1*Console (common LAY with RS232/485 pins), TF card |
| Expansion Interfaces/Features | - External TPM2.0 is optional, not available by default. Default is CPU built-in TPM2.0 |
|  | - 1*M.2 E-Key (PCIe+USB2.0 protocol, WIFI/BT module) |
|  | - 1*RS232/RS485 (hardware loading and BIOS setting switching) row of pins, pitch 2.0mm |
|  | - 1*USB2.0 Pin, 2x5Pin, 2.0mm pitch; 8 bit GPIO Pin, 2.0mm pitch |
|  | - 1*4Pin PWM CPU FAN |
| Power Supply | - DC 12-19V, 120W or above |
| Operating Environmen | - Operating Temperature: -20℃ ~ +60℃; Operating Humidity: 5% ~ 90 |
|  | - Storage Temperature: -40℃ ~ +85℃; Storage Humidity: 5% ~ 90% |
| Operating System Support | - Windows 10, Windows 11, Linux |
| Dimension | - 120x120 mm |
| Net Weight | - Approx. 130g without heat sink, 250g with heat sink |

AD600NA6L（R100）

Product Size                                         Unit：mm

Order Information

| Module | Optional items | Description of options |
| --- | --- | --- |
| AD600NA6L | - Onboard Intel 12th/13th Generation -U/-P Series Processors | 12th Generation -U/-P Series Processors 13th Generation-U/-P Series Processors |
|  | TPM2.0 | External TPM2.0 optional |
|  | RS232/485 | Hardware material, BIOS setting switch |
|  | M.2 2280 SSD protocol away from memory | PCIe4.0 x4 and SATA3.0 protocol optional, jump resistor |