PX700CA（R100）

Product Feature:

- Onboard AMD RYZEN 6000/7000 series processors

- 2*DDR5 SO-DIMM, total max 64GB

- 2*RTL8125B network

- TPM2.0, M.2 SSD, M.2 WIFI

- 2*HDMI2.0,2*Type-C (USB3.0+DP)

- DC 12-19V; Size: 100x100mm

Specification

| Processor System | - AMD RYZEN 6000/7000 series processor on board, TDP 35W |
| --- | --- |
|  | - EFI BIOS |
| Memory | - 2*DDR5 SO-DIMM, total max. 64GB |
| Storage | - 1*M.2 M-Key 2280(NVMe PCIe 4.0 x4 protocol) |
|  | - 1*SATA3.0 port, 2Pin 5V power supply |
| Display | - 2*HDMI2.0 interface, support 4K@60Hz |
|  | - 2*Type-C interface, support DP 8K@60Hz和USB3.0 |
| Board Side I/O Interface | - Power on button, 2-in-1 audio jack, reset button, 2*USB3.0 |
|  | - DC JACK, 2*HDMI, 2*Type-C |
|  | - Dual Network (RTL8125B) |
| Expansion Interfaces/Features | - External TPM2.0 optional, not available by default. Default is CPU built-in TPM2.0 |
|  | - 1*M.2 E-Key (PCIe+USB2.0 protocol, WIFI/BT module) |
|  | - 1*RS232 pin, pitch 2.0mm |
|  | - 1*USB2.0 Pin, 2x5Pin, 2.0mm pitch |
|  | - 1*4Pin PWM CPU FAN |
|  | - 1*FPC (PCIe4.0 4x) |
|  | - 1*CIR (not available by default, reserved function, software debugging required) |
| Power Supply | - DC 12-19V, 90W and above |
| Operating Environment | - Operating Temperature: -20℃ ~ +60℃; Operating Humidity: 5% ~ 90 |
|  | - Storage Temperature: -40°C ~ +85°C; Storage Humidity: 5% ~ 90% |
| Operating System Support | - Windows 10, Windows 11, Linux |
| Dimension | - 100x100 mm |
| Net Weight | - Approx. 120g without heat sink, approx. 220g with heat sink |

PX700CA（R100）

Product Size                                         Unit：mm

Order Information

| Module | Optional items | Description of options |
| --- | --- | --- |
| PX700CA | - AMD RYZEN 6000/7000 series processor | 6600H/ 7735U/7535U/7735HS |
|  | TPM2.0 | External TPM2.0 optional |