AD742A (R100)

Features：

- Onboard Intel Alder Lake-N series processors;

- Onboard LPDDR5 memory, total maximum 16GB;

- 1 x Intel i226-V network, POE for option;

- 1 x LVDS/EDP, 1 x HDMI, M-Key, E-Key, B-Key;

- 2 x COM, 8_bit GPIO

- DC 12V; Size: 100x72mm

Product Specifications

| Processor System | Onboard Intel Alder Lake-N series processor, with or without fan |
| --- | --- |
|  | EFI BIOS |
| Memory | Board-mounted LPDDR5 memory slot, up to 16GB |
| Storage | M-Key 2242, PCIe4.0 x2 protocol |
|  | 1 x SATA3.0, 2Pin power supply |
| Display | 1 x HDMI2.0, 4K @60Hz |
|  | 1 x LVDS/EDP |
| Board Edge I/O | DC JACK, HDMI, 2 x USB3.2, 1 x LAN, Audio |
| Extended interface/function | External TPM2.0 is optional, not available by default |
|  | 1 x E-Key (PCIe+USB2.0 protocol, WIFI/BT module) |
|  | 1 x B-Key (USB2.0+ USB 3.0 protocol, 4G/5G module) |
|  | 2 x COM, RS232 /422/485, pitch 2.0mm, BIOS selection |
|  | 2 x JUSB2.0 pin header, 2x5Pin, pitch 2.0mm |
|  | 8_Bit GPIO |
|  | 1 x 4Pin PWM CPU FAN |
| Power Supply | DC 12V, 60W |
| Working Environment | Working temperature: -20℃ ~ +60℃; Working humidity: 5% ~ 90% |
|  | Storage temperature: -40℃ ~ +85℃; Storage humidity: 5% ~ 90% |
| Operating System | Windows10, Windows11, Linux |
| Size | 100 x 72 mm​ |
| Net Weight | Approximately 100g without radiator, approximately 220g with radiator |

AD742A (R100)

Measurement                                                 Unit：mm

Ordering Information

| Model | Options | Descriptions |
| --- | --- | --- |
| AD742A | Onboard Intel Alder Lake-N series processor | N100/N305, with or without fan optional |
|  | TPM2.0 | External TPM2.0 optional |
|  | LVDS/EDP optional | LVDS/EDP optional |
|  | POE optional​ | POE optional​ |